HEALTH & SAFETY DATA SHEET

- PRODUCT IDENTIFICATION

COMMERCIAL NAME: XANTO WAX & SHINE FURNITURE POLISH

PRODUCT TYPE:      Aerosol Polish with Beeswax for wood surfaces

CONTAINER:	          525ml aerosol

CLASSIFICATION:   NOT CLASSIFIED under the UK CHIP Regulations 1994. EXTREMELY

FLAMMABLE under the EC Aerosol Directive.

VARIANTS:                Natural Original, Lavender Lustre, Forest Fruit Daiquiri

NAME & ADDRESS OF SUPPLIER:

The Vitale Group Ltd

34 Gawer Park

Chester

CH1 4DA

England

Tel:  +44 (0) 1244 370444

Fax:  +44 (0) 1244 370555

- COMPOSITION / INGREDIENT INFORMATION

GENERAL DESCRIPTION: A SILICONE/WAX EMULSION IN WATER/WHITE SPIRIT.

THE PROPELLANT IS BUTANE.

Chemical Identity: Aqua, white spirit, cetostearyl alcohol, sodium benzoate, sodium nitrite, sorbitan oleate, PEG(20) sorbitan oleate, alkyl imidazoline methosulphate, alkyl dimethyl benzyl ammonium chloride, beeswax, silicone, parfum.

Substances listed in CHIP or EH40 Classified

Concentration	OES		MEL	R/S phrases	     Symbol

BUTANE 40		<30% w/w	600 ppm	  --          R12, S2, 9, 16, 33.        Fx

WHITE SPIRIT		<10% w/w	1000mg/cu.m	  --	R10, 52/53,65;S2,23,    Xn,N

S24/25,43,61,62.

ALKYLDIMETHYL	<0.1% w/w	  --		  --	R22,34;S26,28,36/37/    C

BENZYLAMMONIUM CHLORIDE 				39,45.

ALKYLIMIDAZOLINE	<0.1% w/w	  --		  --	R36,38;S26,28,37/39     Xi

METHOSULPHATE

- HAZARDS IDENTIFICATION

Eye Contact:		Contact may cause mild irritation

Skin Contact:		Low hazard

Inhalation:		High levels of vapour may cause dizziness

Ingestion:		Accidental ingestion is an unlikely event

Cans may rupture if heated to excess.

- FIRST AID MEASURES

Eye Contact:		Contact lens should be removed.  Irrigate copiously with clean, fresh water

for at least 10 minutes, holding the eyelids apart, and seek medical advice.

Skin Contact:		Wash with water - normal hygiene procedures

Inhalation:		Remove to fresh air, keep warm, seek medical advice

Ingestion:		If accidentally swallowed:  Drink 2-3 glasses of milk or water.  Keep at

rest.  Seek medical advice.  Do NOT induce vomiting.

- FIRE FIGHTING MEASURES

Extinguishing media:		Dry powder, foam & CO2 	may be used.

Special fire fighting procedures:	Fire exposed containers should be sprayed with water to lessen the

risk of explosion.

- ACCIDENTAL RELEASE MEASURES

If containers are ruptured extinguish all ignition sources. Ventilate area, particularly at low level. Absorb residue on inert material and dispose of in accordance with local regulations.

- HANDLING AND STORAGE

Store at ambient temperatures under dry conditions.

Read the label before use.

Do not mix with other chemicals.

Keep out of the reach of children.

- EXPOSURE CONTROLS / PERSONAL PROTECTION

Protective clothing is required. Wear gloves and avoid eye contact by wearing eye protection if necessary.

Do not smoke during use.

Ensure adequate ventilation.

Wash hands after use.

- PHYSICAL AND CHEMICAL PROPERTIES

Appearance			Off white emulsion

PH				7.8

Internal pressure at 20C		Approximately 3 bar

Vapour Density			>Air

Odour				As per fragrance

Flammability			See section 16

Solubility in water at 20 C		Insoluble

Relative Density			Approximately 0.7

- STABILITY AND REACTIVITY

The aerosol is stable under normal conditions.

Cans may  rupture in a fire with the release of flammable propellant.

The product must not be used on copper, aluminium or bright metal.

- TOXICOLOGICAL INFORMATION

The product under normal conditions of storage and use is not likely to present a health hazard.

Ingestion is unlikely.

- ECOLOGICAL INFORMATION

Most of the chemicals used are biodegradable.

No chlorofluorocarbons (CFC’s) are present.

White spirit is slightly toxic to aquatic organisms, but will slowly degrade.

- DISPOSAL CONSIDERATIONS

Do not puncture or incinerate even when empty.

Dispose of in accordance with local regulations.

- TRANSPORT INFORMATION

IMDG		Class; “Aerosols”		UN Code	1950

ADR		Class; “Gases”		Marine Pollutant	 No

OTHER INFORMATION; Cannot be transported by air.

- REGULATORY INFORMATION

The labelling is in accordance with both the “Chemical (Hazard Information and Packaging for Supply) Regulations 1994” as amended in 1996, and current EC guidelines.

The aerosol carries a statutory aerosol statement (see section 16) and is labelled EXTREMELY FLAMMABLE.

- OTHER INFORMATION

Aerosol caution.

The container is pressurised. Do not pierce or burn, even when empty.

Protect from sunlight and do not expose to temperatures exceeding 50 C.

Do not spray on a naked flame or any incandescent material.

Keep out of reach of children.

Keep away from sources of ignition – no smoking.

This information is based on our present state of knowledge and is intended to describe the product from the point of view of safety. It should not be construed as a guarantee of specific properties. No responsibility can be accepted for the consequences of misuse of the product.